Ultimate 20 Tuning Guide

Full power w/ Open leeches

BASE 								22 UPPERS

4-7 knots								18 LOWERS

Driver only on high side						(15 Headstay)

No Cunningham, No Vang, Outhaul slightly eased just past a fist width

Mainsheet eased approx 8”-12” from MAX mark

Traveler car Up to highside footbrace

Full power w/ Tight leeches

STEP UP 1			+3 Uppers  		25 UPPERS

8-12 knots			+2 Lowers		21 LOWERS

All crew high side 							(19 Headstay)

Outhaul snug, fist width to boom, No Vang, Cunningham cleated snug

Mainsheet from 5” eased to MAX mark

Traveler car Up varies from center to highside footbrace

Over powered / Depower

STEP UP 2			+3 Uppers		28 UPPERS

13-18 knots			+2 Lowers		23 LOWERS

All crew fully hiking						(22.5 Headstay)

Outhaul tight to boom, Cunningham ½ on

No Vang until Mainsheet has to be eased

Mainsheet from 2” eased to MAX mark

Traveler car played continuously from Center to Leeward cockpit

STEP UP 3 			+2 Uppers		30 UPPERS

19+ knots				+2 Lowers		25 LOWERS

Outhaul back to snug, Cunningham tight down, Vang on, but not max

Mainsheet from MAX mark to 4” eased

Traveler around center if Vang sheeting begins

Tuning guide based on the aluminum Ballenger Mast with 5/32” 1x19 or 4mm Dyform shrouds.

The BASE Setting is set for Light Air conditions.  Drifter cond. 19-13 (11 Headstay)

All settings are approximate and mainly depend on sea state and puffs.

Mainsheet MAX mark should be a piece of tape or Sharpie placed on the mainsheet line somewhere between the last purchase and the mainsheet cleat.  The MAX mark should be set when mainsail is fully trimmed, but not sheeted so hard as to have wrinkles from the clew to the lower third of the mast.